| Table 4. SU, KW and ER values for multiple experiments |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | S.no | pressure (MPa) | stand of distance (mm) | traverse speed (mm/min) | Mass flow rate (kg/min) | Surface Undulation (µm) | Erosion Rate(mm³/min) | Kerf Width(mm) |
|  | 1 | 220 | 2 | 80 | 0.4 | 4.75 | 119.98 | 2.08 |
|  | 2 | 260 | 1 | 64 | 0.55 | 3.09 | 126.36 | 1.44 |
|  | 3 | 220 | 2 | 80 | 0.45 | 4.46 | 122.4 | 1.98 |
|  | 4 | 140 | 3 | 96 | 0.55 | 5.02 | 111.57 | 2.25 |
|  | 5 | 300 | 1 | 96 | 0.35 | 3.84 | 126.63 | 1.75 |
|  | 6 | 140 | 1 | 64 | 0.35 | 5.04 | 97.63 | 2.25 |
|  | 7 | 220 | 2 | 72 | 0.45 | 4.38 | 121.6 | 1.95 |
|  | 8 | 140 | 3 | 64 | 0.55 | 4.61 | 109.81 | 2.09 |
|  | 9 | 280 | 2 | 80 | 0.45 | 4.14 | 137.08 | 1.83 |
|  | 10 | 300 | 1 | 64 | 0.35 | 3.5 | 117.28 | 1.59 |
|  | 11 | 300 | 3 | 96 | 0.35 | 3.97 | 143.3 | 1.86 |
|  | 12 | 220 | 2 | 88 | 0.45 | 4.63 | 125.7 | 2.04 |
|  | 13 | 140 | 3 | 96 | 0.35 | 5.56 | 106.95 | 2.57 |
|  | 14 | 300 | 3 | 96 | 0.55 | 3.62 | 148.41 | 1.64 |
|  | 15 | 140 | 1 | 64 | 0.55 | 4.08 | 102.02 | 1.86 |
|  | 16 | 180 | 2 | 80 | 0.45 | 4.75 | 115.2 | 2.15 |
|  | 17 | 220 | 2 | 80 | 0.45 | 4.46 | 122.4 | 1.98 |
|  | 18 | 220 | 1.5 | 80 | 0.45 | 4.41 | 119.69 | 1.96 |
|  | 19 | 300 | 3 | 64 | 0.55 | 3.42 | 145.92 | 1.52 |
|  | 20 | 220 | 2 | 80 | 0.5 | 4.27 | 131.74 | 1.93 |
|  | 21 | 220 | 2.5 | 80 | 0.45 | 4.67 | 132.71 | 2.05 |
|  | 22 | 140 | 1 | 96 | 0.35 | 5.36 | 99.7 | 2.43 |
|  | 23 | 300 | 1 | 96 | 0.55 | 3.29 | 140.98 | 1.48 |
|  | 24 | 220 | 2 | 80 | 0.45 | 4.46 | 122.4 | 1.98 |
|  | 25 | 140 | 1 | 96 | 0.55 | 4.53 | 104.26 | 2.08 |
|  | 26 | 140 | 3 | 64 | 0.35 | 5.23 | 103.66 | 2.34 |
|  | 27 | 300 | 3 | 64 | 0.35 | 3.80 | 139.85 | 1.73 |

Table 5: Hyperparametrs for optimized models.

| Output | SVR Hyper parameter | SVR R² | Elasticnet Hyper parameter | EN R² | XGB Hyper parameter | XGB R² | NN Hyper parameter | NN R² |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Surface Undulation | {'C': 10, 'kernel': 'rbf', 'gamma': 'scale'} | 0.981 | {'alpha': 0.1, 'l1_ratio': 0.5} | 0.887 | {n_estimators=100, max_depth=5, learning_rate=0.05} | 1.000 | {'hidden_layer_sizes': architecture=(6,4,3), alpha=0.001, learning_rate=0.0} | 0.994 |
| Erosion Rate | {'C': 10, 'kernel': 'rbf', 'gamma': 'scale'} | 0.888 | {'alpha': 0.1, 'l1_ratio': 0.5} | 0.939 | {'n_estimators':100, 'max_depth':5, 'learning_rate':0.05, 'subsample':0.8} | 0.999 | {'hidden_layer_sizes:' architecture=(8,5,4), alpha=0.001, learning_rate=0.01} | 0.915 |
| Kerf Width | {'C': 10, 'kernel': 'rbf', 'gamma': 'scale'} | 0.913 | {'alpha': 0.1, 'l1_ratio': 0.5} | 0.813 | {'n_estimators':100, 'max_depth':5 ,'learning_rate':0.05, 'subsample':0.8} | 0.999 | {'hidden_layer_sizes': architecture=(6,3,2), alpha=0.001, learning_rate=0.01} | 1.000 |

Fig. 7. Validation of each model with experimental value

Table 6: Ensemble Model performance.

| Ensemble Model Performance Summary |  |  |  |
| --- | --- | --- | --- |
| Target | RMSE | MAE | R² |
| Surface Undulation | 0.0706 | 0.0593 | 0.9872 |
| Erosion Rate | 1.4826 | 1.1811 | 0.9894 |
| Kerf Width | 0.0437 | 0.0363 | 0.9751 |

SURFACE UNDULATION:

(a)                                                                 (b)                                                        (c)

Fig. 8. Ensemble Surface Undulation Model Performance (a) Training and Validation Curve (b) Actual vs Predicted value (c) Density curve (d) Heat map.

Fig. 11. Learning curve for Cross-validation Surface Undulation.

Erosion Rate

Fig. 8. Ensemble Erosion Rate Model Performance (a) Training and Validation Curve (b) Actual vs Predicted value (c) Density curve (d) Heat map.

Fig. 11. Learning curve for Cross-validation Erosion Rate

KERF WIDTH

Fig. 8. Ensemble Kerf width Performance (a) Training and Validation Curve (b) Actual vs Predicted value (c) Density curve (d) Heat map.

Fig. 11. Learning curve for Cross-validation Kerf Width.

Distribution:

| Table 3: Optimal Process Parameter Settings for Different Objectives |  |  |  |
| --- | --- | --- | --- |
| Parameter | Min SU | Max ER | Balanced |
| Pressure (MPa) | 260.00 | 300.00 | 300.00 |
| Standoff Distance (mm) | 1.00 | 3.00 | 3.00 |
| Traverse Speed (mm/min) | 64.00 | 96.00 | 96.00 |
| Mass Flow Rate (kg/min) | 0.55 | 0.55 | 0.55 |
| Expected SU (μm) | 3.22 | 3.67 | 3.67 |
| Expected ER (mm³/min) | 126.48 | 148.81 | 148.81 |

| Table 3: Optimal Process Parameter Settings for Different Objectives |  |  |  |
| --- | --- | --- | --- |
| Parameter | Min SU | Min Kerf | Balanced |
| Pressure (MPa) | 260.00 | 260.00 | 260.00 |
| Standoff Distance (mm) | 1.00 | 1.00 | 1.00 |
| Traverse Speed (mm/min) | 64.00 | 64.00 | 64.00 |
| Mass Flow Rate (kg/min) | 0.55 | 0.55 | 0.55 |
| Expected SU (μm) | 3.22 | 3.22 | 3.22 |
| Expected Kerf(mm) | 1.52 | 1.52 | 1.52 |

| Table 3: Optimal Process Parameter Settings for Different Objectives |  |  |  |
| --- | --- | --- | --- |
| Parameter | Min Kerf | Max ER | Balanced |
| Pressure (MPa) | 260.00 | 300.00 | 300.00 |
| Standoff Distance (mm) | 1.00 | 3.00 | 3.00 |
| Traverse Speed (mm/min) | 64.00 | 96.00 | 96.00 |
| Mass Flow Rate (kg/min) | 0.55 | 0.55 | 0.55 |
| Expected Kerf (mm) | 1.52 | 1.68 | 1.68 |
| Expected ER (mm³/min) | 126.48 | 146.81 | 146.81 |

Table 3: Optimal Process Parameter Settings for Different Objectives

BOOSTRAP

HYBRID Ensemble